Following a disaster, each water agency should initiate a preliminary damage assessment of their facilities. The following steps may help perform a damage assessment.

As soon as possible for major water facilities (and later for all other facilities), each agency should:

- Determine the need to repair, replace, or abandon a facility.
Estimate the cost(s) to restore a facility.

- Consider the potential effects of aftershocks and, if appropriate, evacuate facilities in danger of collapse, and notify local authorities.

- Mobilize field crew(s) to close and tag damaged facilities and equipment.

The following FACILITIES DAMAGE CHECKLIST summarizes the facilities to be evaluated.

SURFACE RESERVOIRS

______	Check for seepage, leaks, cracks, landslides, embankments slump, broken inlet-outlet pipes, piezometer, underdrains.

______	Notify State Division of Dams (through WEROC to MARS network) if
problems are encountered.

______	Lower water levels to reduce the possibility of structural failure.

WELLS

______	Check for power disconnect.

______	Test for contamination.

______	Check for failure of pumps or motors.

______	Check for physical damage.

TREATMENT PLANTS

______	Check if power is on and condition of mechanical and electric equipment.

______	Check quality of outflow.

______	Check for chemical spills or release.

______	Check the need for emergency purification.

This model form/template must be customized to meet your Agency’s needs.

______	Check for structural damage.

TANKS

______	Check for evidence of failure of sub-base

______	Check for leaks, cracks, broken inlet-outlet pipes, underdrains

______	Check for buckling

PUMPING AND GENERATING PLANTS

______	Check transformers for damage and test capacity

______	If generators are water cooled, check for adequate water storage and provide make up water

______	Check for power disconnect

______	Check for structural damage

PIPES

______	Check air and vacuum valves

______	Check for leaks, breaks, pressure loss in lines, cross-connections between water and sewage, overflow into streets, watercourses

______	Check mechanical couplings

Source: California Department of Water Resources, 1984