Standard Operating Guideline

Canal Safety

District Name: ________________________________________

Date Prepared:			   Date Revised:

OBJECTIVE:

- Ensure effective system-wide preventive and predictive maintenance actions.

- Ensure periodic task accomplishment.

- Identify potential/actual system problems.

- Maintain optimum water flow/service to water customers.

RATIONALE/PURPOSE:

- Ensure mission capability.

- Ensure system reliability.

- Develop predictive maintenance programs.

- Determine capital improvement budgeting.

- Develop loss trend analysis.

- Reduce property, liability, and injury/illness loss exposures.

- Reduce revenue loss/or O&M cost.

METHODS/PROCEDURES:

(Note: Listed procedures are intended as guidelines only and may not be applicable for all districts or situations).

Safety

- Install and maintain “No Trespassing” signs, gates, and fences to restrict unwanted public access where practical. Allow landowner access only where required or necessary.

- Post proper hazard signage if razor wire is used on access gates and fences.

This model form/template must be customized to meet your Agency’s needs.

- Install and maintain existing ladders, ropes, and buoys inside flowing canals to allow safe exit from water flows.

- Install and maintain posts and guard rails to prevent cars from driving in the canal.

Community Outreach

- Conduct public awareness campaigns at schools and community events.

Risk Control Best Practices

- Maintain hazard signs at siphons.

- Install screens at siphons.

- Install pipes on top of energy dissipater to indicate underwater hazards.

- Use backhoes or similar equipment to reduce hazardous manual activity such as debris or board removal.

- Avoid driving on wet or otherwise slippery canal banks.

- At least one U.S. Coast Guard-approved 30-inch life ring, with not less than 90-feet of 600-pound capacity line attached, shall be kept in a conveniently accessible place where employees work exposes them to the hazard of drowning, or each employee so exposed shall wear a U.S. Coast Guard-approved personal flotation device.

- A second rope, suitable for securing a rescuer should also be in place. Each rope should be 150-feet long and 600-lb capacity. In the event of a rescue, the second person out of the water should be connected to the primary rescuer by means of the second rope.

- Any personal flotation device shall be approved by the United States Coast Guard as a Type I PFD, Type II PFD, Type III PFD, or their equivalent, and maintained in good condition.

- Single employees pulling debris from canals shall remain behind the guard rails.

- After dark, two or more employees will work together in removing trash from canals.

- Heavy accumulation of debris in the water shall be removed by a backhoe or other mechanized equipment, if possible.

Fall Protection

(As applicable for canals with high velocity flows, deep water, or other safety concerns or risks).

- Install guard rails on elevated areas over water to allow handholds and fall protection anchor points. Guardrails should be installed over walkways, 42-inches high with a mid-rail.

- Fall arrest anchors in the form of static lines or rails with trolleys are the preferred method of attachment to allow movement.

- If lanyards are used, they shall be adjusted to prevent falls and entry into the water.

- Use additional retrieval lines and winch (or other means of retrieval) shall be used. All equipment shall be approved for such use.

- Install slip proof decking on elevated areas over water to reduce the risk of slipping.

- Employee’s feet shall remain in contact with the walkway at all times when removing debris from the water. If guard rails are not present, employees shall use a fall restraint or fall arrest system with a backup person present.

- In dry canals, fall protection shall be provided at distances over 6-feet from the bottom of the canal.

INCIDENT RESPONSE: See Canal Failure Response SOG

- Call 911 for drowning or severe injuries.

- Administer first aid and CPR.

SAFETY CONSIDERATIONS:

An effective maintenance program can help to prevent damage to property, the environment, and injury to employees and the public, and will help to:

- Preclude system and equipment damage.

- Preclude property and environmental damage.

- Facilitate operations and maintenance personnel safety.

Identification of safe work practices among employees should include:

- Train employees and document.

- Fall protection equipment use, maintenance, and inspection.

- Traffic control.

- Canal safety.

- Proper footwear.

- CPR and first aid training.

- Safe crane and heavy equipment operation.

- Crane inspection and certification (if required).

- Crane operator training and certification (if required).

COST/ BENEFIT:

- Reduce revenue losses.

- Preventive maintenance and loss trend data would reduce facility, equipment, and system failures.

- Avoid costly liability, property, and injury/illness losses.

- Create a manageable capital improvement budget to support effective O&M practices.

INSPECTION FORMS/CHECKLISTS/DOCUMENTATION:

- Training Log.

- Canal Structure Inspection Form.

REFERENCES:

- U.S. Bureau of Reclamation, Canal Operators Manual, January 2018

- U.S. Bureau of Reclamation, Canal Operation, and Maintenance: Embankments, November 2017

- U.S. Bureau of Reclamation, Canal Safety Program

- U.S. Army Corps of Engineers, Levee Owner’s Manual for Non-Federal Flood Control Works, March 2006, Section 2.6.

- CCR, Title 8, Section 3389 (Life Rings and Personal Flotation Devices)

- Cal-MUTCD – Traffic Control Manual/Procedures